FBMH Safety Guidance on Storing of Peroxide Forming Solvents – applies to pure materials only	 (Adapted from Merck “Peroxide Forming Solvents”)

A significant number of laboratory solvents can undergo autoxidation under normal storage conditions to form unstable and potentially dangerous peroxide by-products. This process is catalysed by light and heat and occurs when susceptible materials are exposed to atmospheric oxygen.

Storage times indicated below are based on these solvents continually being stored at in opaque containers, appropriate temperatures and under inert atmospheric gases. Exposure to air and light may accelerate the formation of explosive peroxides.

Potential peroxide-forming solvent chemicals are classified into 2 groups

Group A:   Chemicals that form explosive levels of peroxides without concentration

- Isopropyl ether

Purchase

You must notify your Compliance and Risk Manager in advance of purchase

Purchase in limited quantities only

Storage

Write date of arrival on the container, date it needs disposing by (i.e. 3 months after arrival) and name of principle investigator

Ensure you have a storage SOP in place and attach it to your COSHH Risk Assessment

The SOP must include emergency planning i.e. actions to be taken in the event of a peroxide test being positive and or any visual changes.

Records of peroxide testing must be kept

Discard after 3 months in storage

Peroxide Testing

Test for peroxide formation before use (see below for testing methods)

Completed the relevant risk assessments

Group B:  Solvents that are peroxide hazards on concentration and long term storage

Note: These are the most common substances - materials other than those listed may form peroxides.

| - Acetal | - Furan |
| --- | --- |
| - Acetaldehyde | - 4-Heptanol |
| - Benzyl Alcohol | - 2-Hexanol |
| - 2-Butanol Dioxanes | - Methyl Acetylene |
| - Chlorofluoroethylene | - 3-Methyl-1-butanol |
| - Cumene(isopropylbenzene) | - Methyl-isobutyl ketone |
| - Cyclohexene | - 4-Methyl-2-pentanol |
| - 2-Cyclohexen-1-ol | - 2-Pentanol |
| - Cyclopentene | - 4-Penten-1-ol |
| - Decahydronapthalene(decalin) | - 1-Phenylethanol |
| - Diacetylene(butadiyne) | - Tetrahydrofuran |
| - Dicyclopentadiene | - Tetrahydronapthalene |
| - Diglyme | - Vinyl Ethers |
| - Diethyl ether | - Sec. Alcohols |
| - Ethylene glycol ether acetates |  |

Purchase

Purchase in limited quantities only

Storage:

Write date of arrival on the container, date it needs disposing by (i.e. 1year after arrival) and the name of principle investigator

Ensure you have a storage SOP in place and attach it to your COSHH Risk Assessment

The SOP must include emergency planning i.e. actions to be taken in the event of a peroxide test being positive and or any visual changes.

Records of peroxide testing must be kept

Discard after 1year in storage

Peroxide Testing (see below for testing methods)

Test for peroxide formation before distillation or evaporation.

Test for peroxide formation after 1 year.

General Handling Considerations for Peroxidizeable Solvents

- All peroxide-forming solvents should be checked for the presence of any peroxides prior to distillation or evaporation.

- Solvents containing low levels of free radical scavengers such as BHT (known as inhibitors) should be used whenever the presence of the stabilizing species does not interfere with intended application.

- Uninhibited materials should be stored with care and frequently checked for peroxide formation.

- Peroxide-forming solvents should be purchased in limited quantities and strict stock rotation used so older material in the inventory are be used first. Materials should be stored away from light and heat with tightly secured caps and labelled with dates of receipt.

- Periodic testing to detect peroxides should be performed and recorded.

Peroxide Detection

A variety of methods are available to test for the presence of peroxides in organic solvents with the two most common tests described below.

There are a range of visual checks that should be outlined in your risk assessment and storage SOP.  If there are visible crystals, the substance has changed colour, there is visible precipitate or an oily viscous layer present in the material, and these indicate high levels of dangerous high peroxide levels. STOP – do not touch the bottle. Immediately notify your PI and follow your emergency action plan.

Testing for peroxides:

Quantofix® Peroxide Test Strips (Sigma-Aldrich Part # 37206)

In the presence of hydrogen peroxide the test paper turns blue. Quantofix® Peroxide test sticks can also be used for the determination of peracetic acid and other organic and inorganic hydroperoxides. To test for hydroperoxides in organic solvents, the test zone is wetted with one drop of water after evaporation of the solvent.

Interferences:  In the pH range of 2-9, the accuracy of the determination is independent of the pH of the test solution. Buffer strongly acidic solutions with sodium acetate and adjust alkaline solutions to about pH 5-7 with citric acid. False positive results can only be caused by strong oxidizing agents.

Storage:  Avoid exposing the sticks to sunlight and moisture. Store unopened packs in refrigerator (+2 °C to +8 °C). Opened packs: store container in a cool and dry place. Ensure they are disposed of and replaced prior to the use by date.

Potassium Iodide Indicator

Add 0.5-1.0 ml of the sample solvent to an equal volume of glacial acetic acid containing about 0.1 g of sodium iodide or potassium iodide crystals. A yellow colour indicates iodine formation via iodide oxidation by sample peroxide; a brown colour indicates high concentration. A blank determination should be made particularly when colour development is faint since iodide/acetic acid mixtures will, over time, turn a yellow - brown colour due to air oxidation.

A more sensitive variation of the above method adds one drop of a saturated, aqueous starch solution to the sample solution. Starch and iodine combine to form a bright blue complex that is more easily visualized than the yellow colour generated by iodine alone. Dark blue solution colour would be indicative of high peroxide concentrations.

- See more at:

http://www.sigmaaldrich.com/chemistry/solvents/learning-center/peroxide-formation.html#sthash.HHlnUmYN.dpuf

References

1. Jackson, H.L., McCormack, W.B., Rondestvedt, C.S., Smeltz, K.C., and Viele, I.E. Control of Peroxidizable Compounds, J. Chem. Educ., 1970, 46 (3), A175.

2. Kelly, R.J, Review of Safety Guidelines for Peroxidizable Organic Compounds, Chemical Health and Safety, 1996, 3 (5), 28-36.

3. Clark, D.E., Peroxides and Peroxide - Forming Compounds, Chemical Health and Safety, 2001, 8 (5), 12-21.

See more information at: http://www.sigmaaldrich.com/chemistry/solvents/learning-center/peroxide-formation.html#sthash.HHlnUmYN.dpuf